MORSE IP CORE

Detailed Design Description

Project: SoC Morse Decoder with LCD & LEDR Display

Platform: Terasic DE10 Standard (Intel Cyclone V) · Quartus Prime

Author: Trần Hồng Sơn  |  December 2024

Course: TH Thiết kế SoC — Khoa Điện tử Viễn thông, ĐHKHTN TP.HCM

1.  MORSE IP CORE — OVERVIEW

The Morse IP is a custom Avalon-MM slave peripheral implemented in Verilog and integrated into a Nios II SoC via Intel Platform Designer. It accepts Morse-code input, decodes it to an ASCII character, and exposes decoded data to the processor through an Avalon register interface.

Top-level Block Diagram

The IP sits on the Avalon Switch Fabric alongside the Nios II processor, On-Chip Memory, JTAG UART, GPIO (LCD), and the Button PIO:

Module Signature

module Morse_IP #(

parameter DOT_TIME  = 25_000_000,   // 0.5 s at 50 MHz

parameter WAIT_TIME = 50_000_000    // 1.0 s at 50 MHz

) (

// System

input        clk,

input        reset_n,

// Avalon-MM Slave

input        write,

input        read,

input  [1:0] address,

input [31:0] writedata,

output reg [31:0] readdata,

// Conduit

input  [7:0] inital,      // SW[7:0]: {length[2:0], morse_code[4:0]}

input        MODE,         // SW[9]:  0 = setup mode, 1 = button mode

output reg [7:0] ascii_out,

output reg       dot_t,

output reg       wait_t

);

2.  CONCEPT

The IP supports two operating modes controlled by the MODE conduit signal (wired to SW[9] on the board):

Mode 0 — Setup / Manual Mode

- SW[9] = 0.

- The 8-bit conduit input initial is used directly:

  - Bits [7:5] carry the morse code length (3 bits, 1–5 dits/dahs).

  - Bits [4:0] carry the packed morse code (LSB = first symbol sent).

- The combinational decode logic immediately resolves the {length, code} pair to an ASCII character on ascii_out.

- No FSM interaction. Useful for testing the decode table via DIP switches.

Mode 1 — Real-time Button-press Mode

- SW[9] = 1.

- The user presses KEY[1] to enter Morse symbols in real time.

- A 3-state FSM (IDLE → DETECT → RECORD) measures press duration to distinguish dots from dashes.

- When the user stops pressing (WAIT_TIME elapsed), the accumulated code is decoded and ascii_out is updated.

- KEY[3] triggers the Nios II C code to write ascii_out to the LCD.

- KEY[2] resets the code buffer for the next character.

- KEY[0] is the global active-low reset_n.

Timing Parameters

Both parameters are in clock cycles (50 MHz reference clock on DE10 Standard):

| Parameter | Default Value | Time @ 50 MHz | Purpose |
| --- | --- | --- | --- |
| DOT_TIME | 25 000 000 | 0.5 s | Threshold: press < 0.5 s → dot, press ≥ 0.5 s → dash |
| WAIT_TIME | 50 000 000 | 1.0 s | Idle timeout: gap > 1 s → character complete |

3.  SIGNAL & PORT CONNECTION

The Morse IP exposes three interface types: Avalon-MM Slave, Conduit (external I/O), and internal register signals.

3.1  Avalon-MM Slave Ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock (50 MHz) |
| reset_n | input | 1 | Active-low asynchronous reset |
| write | input | 1 | Avalon write strobe |
| read | input | 1 | Avalon read strobe |
| address | input | 2 | Register address (offsets 0–3) |
| writedata | input | 32 | Data from master (Nios II) |
| readdata | output | 32 | Data to master (Nios II) |

3.2  Conduit Ports

| Signal | Direction | Width | Board Pin | Description |
| --- | --- | --- | --- | --- |
| MODE | input | 1 | SW[9] | Operating mode: 0 = setup, 1 = button-press |
| inital | input | 8 | SW[7:0] | Setup mode input: {length[7:5], morse_code[4:0]} |
| ascii_out | output | 8 | LEDR[7:0] | ASCII code of decoded character (also to LCD via C) |
| dot_t | output | 1 | LEDR[8] | Toggles every DOT_TIME cycles (visual indicator) |
| wait_t | output | 1 | LEDR[9] | Toggles every WAIT_TIME cycles (visual indicator) |

3.3  Internal Signal Summary

| Signal | Width | Description |
| --- | --- | --- |
| morse_code | 5 | Accumulated Morse bits (LSB = most recent symbol) |
| length | 3 | Number of symbols received so far (1–5) |
| signal | 2 | Written by CPU: bit[1] = enable/reset, bit[0] = KEY[1] state |
| timer | 26 | General-purpose cycle counter for FSM timing |
| dot_cnt | 26 | Free-running counter for dot_t toggle |
| wait_cnt | 26 | Free-running counter for wait_t toggle |
| state | 2 | Current FSM state (IDLE / DETECT / RECORD) |

4.  CONTROL / STATUS REGISTER MAP

The Morse IP presents four 32-bit registers to the Nios II processor via Avalon-MM. All registers are 32-bit wide; unused upper bits are zero-extended.

| Offset | Name | Access | Bits | Description |
| --- | --- | --- | --- | --- |
| 0 | signal | W | [1:0] | Write: bit[1] = reset/enable (0 → async reset morse_code & length); bit[0] = KEY[1] live value |
| 0 | MODE | R | [0] | Read: current operating mode (mirrors MODE conduit) |
| 1 | morse_code | R | [4:0] | Read: accumulated Morse-code shift register (5 bits) |
| 2 | length | R | [2:0] | Read: current number of symbols entered (0–5) |
| 3 | ascii_out | R | [7:0] | Read: decoded ASCII character for current {length, morse_code} |

Register Detail — signal (offset 0, write)

- Bit 1 (enable): When written 0, immediately resets morse_code and length to 0 (asynchronous via negedge signal[1]).

- Bit 0 (key): The Nios II C code writes the live state of KEY[1] (btn & 0x1) every loop iteration so the FSM sees button events.

Example C write: IOWR(MORSE_HW_0_BASE, 0, (btn & 0x3));

Register Detail — readdata (offsets 0–3, read)

- Offset 0 → {31'b0, MODE}

- Offset 1 → {27'b0, morse_code}

- Offset 2 → {29'b0, length}

- Offset 3 → {24'b0, ascii_out}

5.  FINITE STATE MACHINE (Mode 1)

When MODE = 1 the IP uses a Mealy/Moore 3-state FSM to decode button press durations into Morse symbols. The FSM is sensitive to posedge clk and negedge signal[1] (asynchronous reset).

5.1  State Encoding

| State | Encoding | Description |
| --- | --- | --- |
| IDLE | 2'b00 | Waiting for KEY[1] to be pressed (signal[0] goes low) |
| DETECT | 2'b01 | KEY[1] is held; timer counts press duration |
| RECORD | 2'b10 | KEY[1] released; waiting WAIT_TIME for next press or end of character |

5.2  State Transition Diagram (ASCII Art)

negedge signal[1]  ←─────────────────────────────────────────

(async reset)              master_write_done (char complete) |

↓                                                       |

┌─────────┐   signal[0]=0   ┌──────────┐                     |

│  IDLE   │ ──────────────→ │  DETECT  │ ←──────────────     |

│         │   (btn pressed) │          │     !done & !full   |

│ length++│                 │ timer++  │                     |

└─────────┘                 └──────────┘                     |

↑                            │  signal[0]=1             |

│                            │  (btn released)          |

│                            ↓                          |

│                     ┌──────────┐                      |

│                     │  RECORD  │─────────────────────-┘

│  timer>WAIT_TIME    │          │   if timer>WAIT_TIME

└─────────────────────│  timer++ │   → next char / IDLE

└──────────┘

5.3  Transition Conditions

| From | To | Condition | Action |
| --- | --- | --- | --- |
| — | IDLE | negedge signal[1] (async) | morse_code=0; length=0; timer=0 |
| IDLE | IDLE | signal[0] = 1 (no press) | timer = 0 |
| IDLE | DETECT | signal[0] = 0 (pressed) | length = length + 1 |
| DETECT | DETECT | signal[0] = 0 (still held) | timer = timer + 1 |
| DETECT | RECORD | signal[0] = 1 (released) | if timer < DOT_TIME: append 0 (dot); else append 1 (dash) |
| RECORD | RECORD | timer ≤ WAIT_TIME | timer = timer + 1 |
| RECORD | IDLE | timer > WAIT_TIME (gap too long → new char) | timer = 0 |

5.4  Morse Code Shift-Register Encoding

morse_code is a 5-bit shift register. Each new symbol is appended at LSB using a left-shift with the new bit:

morse_code = {morse_code[3:0], 1'b0};  // dot

morse_code = {morse_code[3:0], 1'b1};  // dash

Length tracks how many shifts have occurred. The decode logic uses length to select the correct slice of morse_code.

6.  COMBINATIONAL DECODE LOGIC

A purely combinational always @(*) block converts {length, morse_code} into the output ASCII byte. The lookup follows the standard international Morse tree (left branch = dot/0, right branch = dash/1).

6.1  Morse Tree (supported characters)

Depth is the number of symbols (length). Left = dot (0), Right = dash (1):

6.2  Decode Table Summary

| Length | Code (binary) | ASCII | Character |
| --- | --- | --- | --- |
| 1 | 0 | 0x45 | E |
| 1 | 1 | 0x54 | T |
| 2 | 00 | 0x49 | I |
| 2 | 01 | 0x41 | A |
| 2 | 10 | 0x4E | N |
| 2 | 11 | 0x4D | M |
| 3 | 000 | 0x53 | S |
| 3 | 001 | 0x55 | U |
| 3 | 010 | 0x52 | R |
| 3 | 011 | 0x57 | W |
| 3 | 100 | 0x44 | D |
| 3 | 101 | 0x4B | K |
| 3 | 110 | 0x47 | G |
| 3 | 111 | 0x4F | O |
| 4 | 0000 | 0x48 | H |
| 4 | 0001 | 0x56 | V |
| 4 | 0010 | 0x46 | F |
| 4 | 0100 | 0x4C | L |
| 4 | 0110 | 0x50 | P |
| 4 | 0111 | 0x4A | J |
| 4 | 1000 | 0x42 | B |
| 4 | 1001 | 0x58 | X |
| 4 | 1010 | 0x43 | C |
| 4 | 1011 | 0x59 | Y |
| 4 | 1100 | 0x5A | Z |
| 4 | 1101 | 0x51 | Q |
| 5 | 00000 | 0x35 | 5 |
| 5 | 00001 | 0x34 | 4 |
| 5 | 00011 | 0x33 | 3 |
| 5 | 01111 | 0x31 | 1 |
| 5 | 11111 | 0x3A | 0 (:) |
| 5 | 11110 | 0x39 | 9 |
| 5 | 11100 | 0x38 | 8 |
| 5 | 11000 | 0x37 | 7 |
| 5 | 10000 | 0x36 | 6 |
| any | unsupported | 0x5F | _ (underscore — invalid) |

7.  SOFTWARE SEQUENCE (Nios II C Code)

The Nios II C application runs a polling loop that bridges hardware (Morse IP + Button PIO) to the output peripherals (LCD + printf):

7.1  Initialization Sequence

- Send LCD command 0x01 — clear display

- Send LCD command 0x38 — 8-bit, 2 lines, 5×8 dots

- Send LCD command 0x0F — display on, cursor on, blink on

- Send LCD command 0x06 — entry mode: increment, no shift

- Send LCD command 0x80 — set DDRAM address to line 1, column 0

- Write static header string "20200331 sOC" (12 bytes from table[])

- Send LCD command 0xC0 — move cursor to line 2

7.2  Main Loop (Polling)

while (1) {

// 1. Read physical button states

unsigned int btn = IORD(BUTTON_BASE, 0);

// 2. Forward button bits 0–1 to Morse IP signal register

IOWR(MORSE_HW_0_BASE, 0, (btn & 0x3));

// 3. Read back decoded values from IP

unsigned int reg_val   = IORD(MORSE_HW_0_BASE, 1); // morse_code

unsigned int bit_val   = IORD(MORSE_HW_0_BASE, 2); // length

unsigned int ascii_code= IORD(MORSE_HW_0_BASE, 3); // ASCII out

unsigned int mode      = IORD(MORSE_HW_0_BASE, 0); // current mode

// 4. KEY[2] (bit 2) = 0 → trigger write to LCD

if (mode == 0 || mode == 1) {

if (((btn & 0b100) >> 2) == 0) {

SendData(ascii_code);  // write char to LCD

printf(...);           // debug print via JTAG UART

}

}

}

7.3  LCD Communication Timing

GPIO[15:0] is used to drive the 16×2 Character LCD. The upper nibble carries control lines; the lower byte carries data:

| GPIO bits | Function | Command write value | Data write value |
| --- | --- | --- | --- |
| [8] | EN (enable strobe) | 0x0100 ¦ cmd → 0x0000 ¦ cmd | — |
| [10] | RS (register select) | 0 (command) | 1 (data) |
| [7:0] | D7–D0 data bus | cmd byte | data byte |
| — | Pulse width | 40 ms delay between EN edges | 40 ms delay |

SendCommand() pulls RS=0; SendData() pulls RS=1 (bit 10 of GPIO word).

8.  SYSTEM INTEGRATION

8.1  Platform Designer Address Map

| Component | Base Address | End Address | Interface |
| --- | --- | --- | --- |
| nios2_gen2_0 | — | — | Nios II/e Processor |
| onchip_memory2 | 0x0000_0000 | 0x0000_FFFF | On-Chip RAM/ROM (64 KB) |
| jtag_uart_0 | 0x0001_1030 | 0x0001_1037 | JTAG UART |
| button | 0x0001_1020 | 0x0001_102F | PIO (KEY[3:1] input) |
| gpio | 0x0001_1010 | 0x0001_101F | PIO (GPIO[15:0] output → LCD) |
| morse_hw_0 | 0x0001_1000 | 0x0001_100F | Morse IP Slave (4 × 32-bit regs) |

8.2  Top-level Pin Assignments (Morse_top.v)

module Morse_top (

input         CLOCK_50,

input  [9:0]  SW,

input  [3:0]  KEY,

output [15:0] GPIO,

output [9:0]  LEDR

);

Morse_hw sys (

.clk_clk                      (CLOCK_50),

.reset_reset_n                (KEY[0]),

.button_external_connection_export (KEY[3:1]),

.gpio_external_connection_export   (GPIO[15:0]),

.morse_hw_0_ascii_out_export       (LEDR[7:0]),

.morse_hw_0_dot_time_export        (LEDR[8]),

.morse_hw_0_wait_time_export       (LEDR[9]),

.morse_hw_0_mode_export            (SW[9]),

.morse_hw_0_setup_export           (SW[7:0])

);

endmodule

9.  EVALUATION & KNOWN LIMITATIONS

9.1  Verified Results

- Mode 0: Correct ASCII decode for length 1–4 verified with DIP switches (G, O, E, W, H, etc.).

- Mode 1: Real-time button press correctly distinguishes dot vs dash at 0.5 s threshold; gap of 1 s correctly triggers next character.

- LCD correctly displays decoded characters on line 2 in sequence.

- LEDR[7:0] shows binary ASCII of current decode; LEDR[8–9] show dot/wait clock indicators.

9.2  Limitations

- Simulation: Hardware relies on many real-time signals (button timing, LCD strobes) making ModelSim testbench impractical without a full stimulus script.

- Character set: The Morse tree covers A–Z, 0–9 and selected punctuation. Vietnamese diacritical characters are not supported.

- Timing parameters: DOT_TIME and WAIT_TIME are compile-time parameters; dynamic adjustment at runtime is not implemented.

- LCD capacity: The 16×2 display fills up after 16 characters on line 2 with no auto-wrap or scroll logic in the current C driver.

9.3  Possible Improvements

- Add a hardware timer (interval timer IP) for more accurate, interrupt-driven timing instead of CPU-polling.

- Implement LCD auto-wrap and two-line character scrolling in the C driver.

- Extend the Morse lookup table to cover all ITU-R M.1677-1 punctuation.

- Add an Avalon interrupt output so the CPU is notified on character completion rather than polling.